Some Nice Paint Formulas for Restorers

by Lars Grimsrud

Colorado Corvette Crazies (CCC)

The Ultimate Corvette Tuning & Beer Drinking Fraternity

Lafayette, CO 								Rev. A 1-12-03

For you weekend restoration fanatics, here are some paint info, paint numbers and formulas I’ve gathered over the years. This stuff comes in really handy when you’re doing detail work and when you want things to look correct, original, and fresh:

1.   Order 3 things from Eastwood:

a.  1 can of “AlumaBlast.”  This stuff is amazing.  It looks EXACTLY like cast aluminum.  Even when you’ve bead-blasted your intake manifold and alternator, and after you’ve Hotsie’ed your tranny, the aluminum never looks just like it did when it was new.  A light coat of AlumaBlast will make the parts look exactly as they should – fresh, new aluminum.  Good stuff.

b.  1 can of “Spray Gray.”  Again – amazing stuff.  It looks just like cast iron.  Use it on your master cylinder, steering box, tie rod ends, and other parts you want to look just like fresh cast iron.  You can’t do without this stuff.

c.  High Temp Stainless Steel Coating.  Brush or spray this onto your exhaust manifolds, and they’ll look brand new – show quality.  The stuff won’t burn off, and will make your manifolds look brand new for years.  I use it on all exhaust components for a lasting good look.

2.   Ever wondered what to do with your crappy-looking power brake booster and other cad-plated parts that you don’t want to spend the $$ and time having re-plated? Here is a lacquer paint that you can shoot through your paint gun that will give you the greenish-hue you’ve been looking for.  You can even use it on carb parts to simulate the carb plating.  Thin it down 50% with thinner and spray it on in light coats.  Do not clearcoat:

PPG Lacquer, “Sauterne Sage Poly”  DDL #43433

3.   How about those silver painted Rally Wheels?  The factory used lacquer, but lacquer on wheels just doesn’t hold up. You can re-shoot them with some good, catalyzed Urethane for a very good quality, lasting finish. Here is the info on both lacquer and enamel for the wheels. This is the correct, factory color:

PPG Lacquer: “Non-Smudge Aluminum”  DDL #8568 (thin 50% with thinner).

PPG Delstar (Mix 8:6:1 Delstar/DTR601 Reducer/DXR80 Catalyst):

“Argent Silver”  DAR #8568

Mixing Formula (1 qt):

DMR 433   416 units

DMR 435   420 units

DXR 495   20 units

DMR 499   160 units

If you want to prime the wheels first, I recommend using PPG DP40LF Epoxy Primer mixed 2:1 with DP402LF Catalyst.  This will give you a bullet-proof  finish.

4.   On to the engine.  I see guys with nice ‘Vettes using paint out of a cheap spray can that they bought at the local parts store.  People with nice ‘Vettes who use spray can paint on their engines should be charged with Vette-abuse, and sentenced to restore Honda Civics for life.  If you want your engine to look perfect, to be the perfect, correct color, and to have a durable, lasting finish and shine, you want to shoot it with Delstar catalyzed urethane enamel.  Here is the factory-correct color code and the whole mixing formula:

PPG Delstar Chevy Engine Orange, DAR #60524 H

Mixing Formula (1 qt, cummulative):

DMR 400   4.0 units

DMR 405   160.0 units

DMR 464   660 units

DXR 495   680 units

DMR 499   1250 units

Mix the paint 8:6:1 Delstar/DTR601 Reducer/DXR80 Catalyst and shoot it with a paint gun.  If you have a new engine back from the machine shop and it’s bare metal, prime the entire engine, block, heads, etc. with PPG DP40LF Epoxy Primer mixed 2:1 with DP402LF Catalyst. You won’t believe how good it will look.

As always, contact me if you need any assistance or if you have any questions.

Here’s another one for you. This one took a bit of research and color matching development, so stash this with your restoration files:

The BACK side of the steel GM Rally Wheels and the rim/bead area was not painted silver.  Rather, the entire back of the wheel and the bead area was painted an odd light grey color.  This color is perfectly matched with the following Delstar mixing formula.  This is the tip-top finishing touch for a set of perfectly restored Rally Wheels:

Lars’ GM Wheel Back Color, PPG Delstar (No color code)

Mixing Formula (1 qt) (cumulative units)
DMR 475   6

DMR 490   106

DMR 400   272

DMR 487   442

DMR 415   447

DMR 476   457

DMR 491   477

DXR 495   497

DMR 499   1000

Mix the Delstar 8:6:1 Delstar/DTR601 Reducer/DXR80 Catalyst and shoot it with a paint gun.